EMBER

Emergency Monitor for Building Evacuation and Rescue

Comprehensive Build Guide — Arduino Nano Edition

24-Hour Hackathon | Team of 3 (EE, CE, CS)

Hardware: Arduino Nano + NRF24L01 + IMU + GPS

# Table of Contents

- 1. Project Overview & Architecture

- 2. Complete Hardware Inventory

- 3. System Architecture Diagram

- 4. Wearable Node — Detailed Build

- 5. Base Station — Detailed Build

- 6. Laptop Dashboard

- 7. Floor Plan Integration

- 8. NRF24L01 Radio — Step-by-Step

- 9. 3D Printed Enclosures

- 10. 24-Hour Schedule (EE / CE / CS)

- 11. Testing & Demo Checklist

- 12. Bonus: ESP-01 WiFi Dashboard

- 13. Future Expansions

- 14. Troubleshooting Guide

# 1. Project Overview & Architecture

## 1.1 What EMBER Does

EMBER is a real-time firefighter tracking system for use inside hazardous buildings. Each firefighter wears a sensor node that broadcasts GPS position and orientation data via NRF24L01 radio to a base station outside the building. The base station forwards this data over USB serial to a laptop running a browser-based dashboard, which displays each firefighter’s location overlaid on an uploaded floor plan image.

## 1.2 Key Design Decision: Arduino Nano as Main Controller

This project uses the Arduino Nano (ATmega328P) as the primary microcontroller on all three units. The ESP-01 (ESP8266) modules available to the team have only 2 GPIO pins, which is insufficient for driving the NRF24L01 radio (5 SPI pins), IMU (2 I2C pins), GPS (1 serial pin), and buzzer (1 pin). The Arduino Nano provides 22 usable I/O pins — more than enough for all peripherals.

The ESP-01 modules are reserved as an optional bonus feature: one ESP-01 at the base station can host a WiFi access point with a built-in web dashboard, allowing any phone to view firefighter status without a USB connection (see Section 12).

## 1.3 Scope Limitations

- No heart rate or biometric sensing.

- No fall detection. IMU is used only for heading/orientation.

- Single-floor demo only. Multi-floor requires a BMP280 barometric sensor (see Section 13).

- No WiFi or internet for core data transport. All node-to-base communication uses NRF24L01 at 2.4 GHz.

- No LCD display. All status information is shown on the laptop dashboard.

## 1.4 System Units

Wearable Node A: Worn by Firefighter 1. Arduino Nano + NRF24L01 + IMU + GPS + buzzer on a mini breadboard inside a 3D-printed enclosure.

Wearable Node B: Identical to Node A for Firefighter 2. Different radio pipe address for identification.

Base Station: Arduino Nano + NRF24L01 + buzzer on a breadboard, connected to laptop via USB. Receives radio data from both wearables, outputs JSON over Serial to the laptop.

Laptop Dashboard: Browser-based HTML/CSS/JS application. Reads serial data via the Web Serial API, displays firefighter positions on an uploaded floor plan.

## 1.5 Communication Flow

Wearable A → (NRF24L01 2.4 GHz) → Base Station Nano → (USB Serial) → Laptop Dashboard

Wearable B → (NRF24L01 2.4 GHz) → Base Station Nano → (USB Serial) → Laptop Dashboard

# 2. Complete Hardware Inventory & Role of Each Component

## 2.1 Master Component List

| Component | Qty Needed | Qty Available | Assigned To | Purpose |
| --- | --- | --- | --- | --- |
| Arduino Nano (ATmega328P) | 3 | 3 | Wearable A, Wearable B, Base Station | Main microcontroller. 16 MHz, 32 KB flash, 2 KB SRAM, 22 I/O pins. Runs all sensor reading, radio communication, and serial output. |
| NRF24L01 (standard) | 3 | 1 confirmed, 2–3 possibly available | 1 per Nano | 2.4 GHz radio transceiver. Communicates via SPI. Wearables transmit; base station receives. |
| IMU / Orientation Sensor | 2 | 1 + 2 obtainable | Wearable A, Wearable B | Provides heading data via I2C. Used to show which direction each firefighter is facing. |
| GPS Module (NEO-6M) | 2 | 1 soldered + 1 unsoldered | Wearable A, Wearable B | Provides latitude/longitude. Communicates via Software Serial at 9600 baud. |
| Active Buzzer | 3 | 3 | All 3 units | Audible alerts. Wearable: recall/danger. Base: signal loss or stationary firefighter. |
| ESP-01 (ESP8266) | 1 (optional) | 3 available | Base Station (bonus) | BONUS ONLY: Hosts WiFi access point + web dashboard for phone-based monitoring. Not part of core system. |
| Breadboard | 3 | Multiple available | All units | Mini breadboards for wearables, full-size for base station. |
| Jumper Wires | ~50 | Available | All units | Connect all components. Short wires for compact wearable builds. |
| USB Cables (Mini-B for Nano) | 2 | Available | Base station + programming | One stays permanently connecting base Nano to laptop. |
| Battery Pack (5V USB) | 2 | Obtain before hackathon | Wearable A, Wearable B | Powers each wearable untethered. Standard USB power bank. |
| 10µF Capacitor | 3 | Check availability | 1 per NRF24L01 | CRITICAL for NRF24L01 power stability. Without it, radio fails randomly. |
| 3D Printer Filament | ~50g | Available | Wearable enclosures | PLA for protective cases. |
| Laptop | 1 | 1 | Dashboard station | Runs browser dashboard. Chrome or Edge required for Web Serial API. |

## 2.2 Arduino Nano — Detailed Role

The Arduino Nano is an ATmega328P-based board running at 16 MHz with 32 KB of flash memory and 2 KB of SRAM. It has 14 digital I/O pins (6 with PWM), 8 analog inputs, and communicates with the computer over USB via a built-in CH340 or FTDI chip.

Key constraints vs ESP32 that affect this project:

- Only one hardware UART (Serial): Pins D0 (RX) and D1 (TX) are shared between USB communication and hardware serial. On the wearables, this UART is used for debugging during development. On the base station, it carries JSON data to the laptop. The GPS module CANNOT use this UART — it must use SoftwareSerial on different pins.

- SPI on fixed pins: D11 (MOSI), D12 (MISO), D13 (SCK) are the hardware SPI pins. The NRF24L01 uses these. Note that D13 also has the onboard LED, which may blink during SPI activity — this is normal.

- I2C on fixed pins: A4 (SDA) and A5 (SCL). The IMU uses these.

- 3.3V output: The Nano has a 3.3V regulator pin that provides up to ~150mA. The NRF24L01 requires 3.3V power. CRITICAL: never connect NRF24L01 VCC to the Nano’s 5V pin — it will destroy the radio module.

- 5V logic on SPI/I2C: The Nano runs at 5V logic, while the NRF24L01 and most IMUs are 3.3V devices. In practice, the NRF24L01 tolerates 5V on its data pins (SCK, MOSI, CSN, CE) and outputs 3.3V on MISO, which the Nano reads correctly. Most IMU breakout boards (MPU6050, MPU9250) include level shifters or are 5V-tolerant. However, if you experience issues, add 1K–2.2K series resistors on the SPI data lines as level shifters.

- 32 KB flash: The combined firmware (NRF24L01 + GPS + IMU + buzzer logic) fits comfortably in ~15–20 KB. The RF24 library is ~5 KB, TinyGPSPlus is ~3 KB, and a basic IMU driver is ~2 KB.

## 2.3 NRF24L01 — Detailed Role

The NRF24L01 is a 2.4 GHz ISM band radio transceiver communicating over SPI. For maximum indoor range with the standard module (no external antenna), use 250 Kbps data rate and maximum transmit power.

- Indoor range: 10–30 metres through walls with the standard module. Sufficient for the ~20m demo.

- Payload: Maximum 32 bytes per packet. Our data struct is ~18 bytes.

- Pipe addresses: Up to 6 reading pipes. We use 2 — one per wearable.

- Power: 3.3V ONLY from the Nano’s 3.3V pin. Add a 10µF capacitor across VCC and GND of the NRF24L01.

## 2.4 IMU — Detailed Role

The IMU provides heading/orientation data via I2C. It is used ONLY for determining which direction the firefighter is facing — NOT for fall detection, altitude, or step counting. If the IMU has a magnetometer (MPU9250, BNO055), heading is derived from the compass. If magnetometer-free (MPU6050), heading is estimated from gyroscope integration (drifts over time but acceptable for a demo).

## 2.5 GPS Module — Detailed Role

The GPS module (NEO-6M) outputs NMEA sentences at 9600 baud. On the Arduino Nano, it MUST use SoftwareSerial (not the hardware UART on D0/D1, which is needed for USB). The TinyGPSPlus library parses NMEA into latitude, longitude, and fix status.

CRITICAL — SoftwareSerial limitation: SoftwareSerial on the Nano can only reliably listen on ONE SoftwareSerial port at a time. Since GPS is our only SoftwareSerial device, this is not a problem. However, SoftwareSerial disables interrupts briefly during receive, which can occasionally cause NRF24L01 packets to be missed. Mitigation: use a high radio retry count (15 retries) so missed packets are automatically re-sent.

Soldering: One GPS module needs header pins soldered on before or at the start of the hackathon.

## 2.6 Buzzer — Detailed Role

Three active buzzers, one per unit. Use active buzzers (which sound when DC voltage is applied). Connect to any free digital pin — set HIGH to buzz, LOW to stop. No PWM needed.

# 3. System Architecture Diagram

┌───────────────────────────┐     ┌───────────────────────────┐

│   WEARABLE NODE A       │     │   WEARABLE NODE B       │

│                           │     │                           │

│  Arduino Nano             │     │  Arduino Nano             │

│   ├─ GPS (SoftSerial D3)  │     │   ├─ GPS (SoftSerial D3)  │

│   ├─ IMU (I2C A4/A5)     │     │   ├─ IMU (I2C A4/A5)     │

│   ├─ NRF24L01 (SPI)      │     │   ├─ NRF24L01 (SPI)      │

│   └─ Buzzer (D6)         │     │   └─ Buzzer (D6)         │

└───────────────────────────┘     └───────────────────────────┘

│  2.4 GHz Radio               │

└────────────┬──────────────┘

│

┌──────────────────────┴────────────────────┐

│         BASE STATION                     │

│                                           │

│  Arduino Nano                             │

│   ├─ NRF24L01 (SPI) ─ receives both     │

│   ├─ USB Serial ───→ Laptop (JSON)       │

│   ├─ Buzzer (D6)                        │

│   └─ [BONUS] ESP-01 on D3 TX            │

└───────────────────────────────────────────┘

│  USB Cable

┌─────────┴──────────┐

│  LAPTOP DASHBOARD  │

│  Chrome / Edge     │

└───────────────────┘

# 4. Wearable Node — Detailed Build Instructions

Build instructions for ONE wearable. Repeat identically for the second, changing only the node ID and NRF24L01 pipe address in firmware.

## 4.1 Components Per Wearable

- 1x Arduino Nano

- 1x NRF24L01 standard module + 10µF capacitor

- 1x IMU (MPU6050, MPU9250, or BNO055)

- 1x GPS module (NEO-6M) with ceramic antenna

- 1x Active buzzer

- 1x Mini breadboard (400 or 170 tie-point)

- ~15 short jumper wires

- 1x USB power bank (small, 5V)

## 4.2 NRF24L01 to Arduino Nano — Wiring

The NRF24L01 uses the Nano’s hardware SPI pins. Wire EXACTLY as follows:

| NRF24L01 Pin | Nano Pin | Notes |
| --- | --- | --- |
| GND | GND | Any Nano GND pin. |
| VCC | 3.3V | CRITICAL: 3.3V ONLY. The Nano’s 3V3 pin. NEVER connect to 5V. |
| CE | D9 | Chip Enable. Controls TX/RX mode. Any free digital pin works; we use D9. |
| CSN | D10 | SPI Chip Select. D10 is the default SS pin on the Nano. |
| SCK | D13 | SPI Clock. Fixed hardware SPI pin. (Onboard LED on D13 may flicker — this is normal.) |
| MOSI | D11 | SPI Master Out Slave In. Fixed hardware SPI pin. |
| MISO | D12 | SPI Master In Slave Out. Fixed hardware SPI pin. NRF outputs 3.3V — Nano reads it fine. |
| IRQ | Not connected | Interrupt pin. Not needed for polling-based operation. |

CRITICAL — Capacitor: Solder or press-fit a 10µF electrolytic capacitor directly across VCC and GND of the NRF24L01 module. Positive leg (longer) to VCC, negative (striped) to GND. Place it as close to the module as possible. Without this capacitor, the radio WILL randomly fail due to voltage drops during transmission. This is the #1 cause of NRF24L01 problems.

## 4.3 IMU to Arduino Nano — Wiring

The IMU uses I2C on the Nano’s dedicated analog pins:

| IMU Pin | Nano Pin | Notes |
| --- | --- | --- |
| VCC | 5V (or 3.3V if module has no regulator) | Most breakout boards (GY-521, GY-91) have onboard regulators and accept 5V. |
| GND | GND | Ground. |
| SDA | A4 | I2C data. Fixed pin on ATmega328P. |
| SCL | A5 | I2C clock. Fixed pin on ATmega328P. |

If the IMU has an AD0/SDO pin, connect to GND for address 0x68 or 3.3V for 0x69. Default (unconnected) is usually 0x68.

## 4.4 GPS to Arduino Nano — Wiring

IMPORTANT: The GPS uses SoftwareSerial, NOT the hardware UART (D0/D1). The hardware UART must remain free for USB serial communication with the computer (debugging during development, and for the base station’s laptop link).

| GPS Pin | Nano Pin | Notes |
| --- | --- | --- |
| VCC | 5V | NEO-6M boards have a 3.3V regulator and accept 5V input. |
| GND | GND | Ground. |
| TX (GPS transmits) | D3 (SoftwareSerial RX) | GPS data output → Nano receives. D3 supports pin-change interrupts needed by SoftwareSerial. |
| RX (GPS receives) | D4 (SoftwareSerial TX) | Nano sends config commands to GPS. Optional but useful for changing update rate. |

Pin choice rationale: D3 is chosen for SoftwareSerial RX because it supports pin-change interrupts (PCINT2), which SoftwareSerial needs to detect incoming data. D2 also supports interrupts but is often reserved for the NRF24L01 IRQ pin if you later decide to use interrupt-driven radio reception. D4 is used for SoftwareSerial TX (any digital pin works for TX).

GPS antenna: The ceramic patch antenna must face upward/outward. When mounting in the enclosure, ensure the antenna side is on top. For indoor demos, acquire initial GPS fix near a window (30–60 second cold start).

## 4.5 Buzzer to Arduino Nano — Wiring

| Buzzer Pin | Nano Pin | Notes |
| --- | --- | --- |
| + (longer leg) | D6 | Digital output. Set HIGH to buzz, LOW to stop. |
| - (shorter leg) | GND | Ground. |

D6 is chosen because it is a PWM pin — if you later want to vary the buzzer tone (for different alert types), you can use analogWrite(). For a simple on/off active buzzer, digitalWrite() is sufficient.

## 4.6 Complete Wearable Pin Map

This table shows every pin on the Arduino Nano and what it connects to. Unassigned pins are available for future expansion.

| Nano Pin | Connected To | Protocol | Notes |
| --- | --- | --- | --- |
| D0 (RX) | USB (via CH340/FTDI) | Hardware UART | Reserved for Serial Monitor / debugging. Do NOT connect anything else here. |
| D1 (TX) | USB (via CH340/FTDI) | Hardware UART | Reserved for Serial Monitor / debugging. |
| D2 | Unassigned |  | Free. Could be used for NRF24L01 IRQ or a button. |
| D3 | GPS TX | SoftwareSerial RX | Receives NMEA data from GPS at 9600 baud. |
| D4 | GPS RX | SoftwareSerial TX | Sends config commands to GPS (optional). |
| D5 | Unassigned |  | Free. |
| D6 | Buzzer + | Digital Output | Active buzzer. HIGH = sound, LOW = off. |
| D7 | Unassigned |  | Free. |
| D8 | Unassigned |  | Free. |
| D9 | NRF24L01 CE | Digital Output | Radio Chip Enable. |
| D10 | NRF24L01 CSN | SPI SS | Radio Chip Select. |
| D11 | NRF24L01 MOSI | SPI MOSI | Radio data out (Nano → NRF). |
| D12 | NRF24L01 MISO | SPI MISO | Radio data in (NRF → Nano). |
| D13 | NRF24L01 SCK | SPI SCK | Radio clock. Onboard LED may flicker. |
| A0–A3 | Unassigned |  | Free analog/digital pins. |
| A4 | IMU SDA | I2C Data | Bidirectional data line. |
| A5 | IMU SCL | I2C Clock | Clock line. |
| A6, A7 | Unassigned | Analog input only | Cannot be used as digital output. Could read battery voltage via voltage divider. |
| 5V | IMU VCC, GPS VCC | Power | 5V power to peripherals with onboard regulators. |
| 3.3V | NRF24L01 VCC + Cap + | Power | 3.3V ONLY for the radio module. Max ~150mA. |
| GND | All GNDs + Cap - | Ground | Common ground for all components. |

## 4.7 Assembly Order

Follow this exact order. Test at each stage before proceeding:

- Mount Nano on breadboard. Push firmly until all pins seat. Verify it powers on via USB (onboard LED lights). Open Arduino IDE, select “Arduino Nano” as the board and the correct COM port. Upload a blank sketch to verify the programming chain works.

- Wire NRF24L01 + capacitor. Connect all 7 pins per Section 4.2. Insert capacitor across VCC/GND. Upload the RF24 library’s “pingPair” or “gettingStarted” example (modified for pins 9, 10). Verify it prints “RF24 initialised” on Serial Monitor. If it prints “RF24 HARDWARE FAIL”, check MOSI/MISO are not swapped and capacitor is present.

- Wire IMU. Connect 4 wires per Section 4.3. Upload an I2C scanner sketch (File → Examples → Wire → i2c_scanner). Verify it prints the IMU address (0x68 or 0x69). Then upload an MPU6050 raw data example — verify accelerometer/gyroscope values appear.

- Wire GPS. Connect 4 wires per Section 4.4. Upload a SoftwareSerial echo sketch that reads from D3 at 9600 baud and prints to Serial Monitor. Take the board near a window. Verify you see NMEA sentences ($GPGGA, $GPRMC). Then upload a TinyGPSPlus example — verify latitude/longitude values appear after fix.

- Wire buzzer. Connect 2 wires per Section 4.5. Upload a sketch that toggles D6 HIGH/LOW every second. Verify you hear beeping.

- Upload combined firmware. Flash the full wearable firmware (reads all sensors, transmits via NRF24L01 every 500ms). Verify data appears on the base station.

- Mount in 3D-printed enclosure. See Section 9.

## 4.8 Data Packet Structure

The wearable transmits this C struct every 500ms. Must be ≤32 bytes:

struct SensorPacket {

uint8_t  nodeId;       // 1 byte  — 1 or 2

float    latitude;     // 4 bytes

float    longitude;    // 4 bytes

float    heading;      // 4 bytes — degrees 0–359.9

uint8_t  gpsFix;       // 1 byte  — 1=fix, 0=stale

uint8_t  alertFlag;    // 1 byte  — 1=distress

uint8_t  batteryPct;   // 1 byte  — 0–100

uint16_t stationaryMs; // 2 bytes — ms stationary

};                       // Total: 18 bytes

Battery reading on Nano: The Nano doesn’t have a built-in battery voltage sensor. To read battery level, create a voltage divider (two resistors, e.g., 100K + 100K) from the battery’s 5V to GND, with the midpoint connected to A0. analogRead(A0) gives a value proportional to battery voltage. Map this to a percentage. For a hackathon demo, you can hardcode batteryPct = 100 and add real battery reading later.

# 5. Base Station — Detailed Build Instructions

The base station is simpler than the wearables — it has NO sensors. It only receives radio data and outputs it over USB. Without an LCD, the Nano’s only peripherals are the NRF24L01 and a buzzer.

## 5.1 Components

- 1x Arduino Nano

- 1x NRF24L01 + 10µF capacitor

- 1x Active buzzer

- 1x Full-size breadboard (830 tie-point, for comfort)

- ~10 jumper wires

- 1x USB cable (Mini-B) permanently connecting to laptop

## 5.2 Base Station Wiring

The base station Nano uses the SAME NRF24L01 pin assignments as the wearables:

| Nano Pin | Connected To | Purpose |
| --- | --- | --- |
| D9 | NRF24L01 CE | Radio chip enable |
| D10 | NRF24L01 CSN | Radio chip select |
| D11 | NRF24L01 MOSI | SPI data out |
| D12 | NRF24L01 MISO | SPI data in |
| D13 | NRF24L01 SCK | SPI clock |
| 3.3V | NRF24L01 VCC + Cap + | Radio power (3.3V ONLY) |
| GND | NRF24L01 GND + Cap − + Buzzer − | Common ground |
| D6 | Buzzer + | Alert buzzer |
| D0 (TX) | USB → Laptop | JSON data to dashboard (via USB cable) |
| D3 (optional) | ESP-01 RX (BONUS) | SoftwareSerial TX to ESP-01 for WiFi dashboard. See Section 12. |

Key difference from wearables: The base Nano’s hardware UART (D0/D1 via USB) carries JSON data to the laptop. This is the primary data link. Serial.println() in the firmware sends data directly to the laptop’s browser dashboard via the Web Serial API. The baud rate is 115200.

## 5.3 Assembly Order

- Mount Nano on breadboard, wire NRF24L01 + capacitor. Identical wiring to the wearable NRF24L01. Flash a receive-test sketch that prints any received data to Serial Monitor.

- Wire buzzer. D6 to buzzer + , GND to buzzer −. Test with a beep sketch.

- Test with wearable: Power on one wearable. Open Serial Monitor on the base Nano. Verify you see received packet data. This is Gate 1.

- Upload final base firmware. Receives packets on dual pipes, deserialises SensorPacket struct, constructs JSON string, prints via Serial at 115200.

## 5.4 Serial Protocol: Base Nano to Laptop

The Nano sends one JSON line per received packet via Serial.println():

{"id":1,"lat":29.6516,"lng":-82.3248,"heading":135.2,"fix":1,"alert":0,"batt":87,"stat":0}

{"id":2,"lat":29.6518,"lng":-82.3250,"heading":270.0,"fix":1,"alert":0,"batt":92,"stat":0}

Bidirectional commands: The laptop dashboard can also write to the serial port. The base Nano listens for incoming serial commands like “BUZZ:1” (alert firefighter 1) or “BUZZ:2” (alert firefighter 2). When received, the Nano briefly switches the NRF24L01 to transmit mode, sends a short command packet to the target wearable, then switches back to receive mode.

# 6. Laptop Dashboard

The laptop dashboard is a single-page browser application (index.html) that runs entirely in Chrome or Edge. It uses the Web Serial API to read data from the base station Nano’s USB serial port. The full code for this dashboard has been provided separately as ember_dashboard.html.

## 6.1 Technology

- HTML5 + CSS3 + Vanilla JavaScript. No frameworks, no build tools, no server.

- Web Serial API for reading/writing serial data in the browser.

- HTML5 Canvas for rendering the floor plan and firefighter markers.

- Works in Chrome and Edge only (Firefox/Safari do not support Web Serial).

## 6.2 Architecture Layers

Serial Layer: Requests a serial port, opens at 115200 baud, buffers bytes into lines, parses each line as JSON. Also exposes a write() function for sending BUZZ commands back.

State Layer: In-memory store holding latest data for each node: position, heading, fix, battery, alert, signal quality, and position trail history.

Canvas Renderer: Draws floor plan image, GPS-to-pixel transformed firefighter markers with heading arrows, smooth position animation (lerp), stale-GPS pulsing, and locate-highlight rings.

UI Panel Layer: Sidebar cards with live stats, battery bars, alert buttons. Top bar with connection controls. Bottom activity log with timestamped events.

Mock Data Generator: Produces fake packets every 500ms so the CS person can develop the entire UI without hardware. Toggle on/off via button. Simulates two firefighters in walking patterns.

## 6.3 Key Interaction: Web Serial → Nano

The Web Serial API baud rate MUST match the Nano’s Serial.begin() rate. Both must be 115200. If they don’t match, the dashboard receives garbled data.

The Nano’s USB serial chip (CH340 or FTDI) requires a driver installed on the laptop. CH340 drivers are available for Windows, macOS, and Linux. If the serial port doesn’t appear in Chrome’s port picker, the driver is missing.

# 7. Floor Plan Integration

## 7.1 Preparing the Floor Plan

- Obtain a floor plan image (architectural PDF, hand-drawn sketch, or screenshot). Convert to PNG or JPG if needed.

- Identify two points on the plan that you can physically visit to record GPS coordinates. The further apart, the more accurate the mapping.

- Visit each point with a phone (Google Maps) and note the lat/lng to 4+ decimal places.

- In the dashboard, upload the image, click Calibrate, click each point on the image, and enter the GPS coordinates.

## 7.2 GPS-to-Pixel Math

The calibration uses a simple linear transform: pixelX = refPx + (longitude − refLng) × pxPerDegLng, pixelY = refPy + (latitude − refLat) × pxPerDegLat. The two calibration points define the scale factors (pxPerDegLat and pxPerDegLng) and the reference offset.

## 7.3 Single Floor Assumption

This version tracks on one floor only. The uploaded plan represents a single floor. Multi-floor tracking requires a BMP280 barometric pressure sensor (see Section 13).

# 8. NRF24L01 Radio Communication — Step-by-Step

## 8.1 Library

Install the RF24 library by TMRh20 in Arduino IDE: Sketch → Include Library → Manage Libraries → search “RF24” → install the one by TMRh20.

## 8.2 Configuration for Max Indoor Range

- Data rate: RF24_250KBPS. Lower rate = higher sensitivity = longer range.

- PA level: RF24_PA_MAX. Maximum transmit power.

- Channel: 108 (above WiFi interference range).

- Retries: setRetries(15, 15). Max delay and max count. Critical because SoftwareSerial on the wearables occasionally disrupts SPI timing.

- CRC: 16-bit (default). Ensures data integrity.

- CE/CSN pins: RF24 radio(9, 10); — this is the constructor call matching our wiring.

## 8.3 Addressing for Two Wearables

// Wearable A firmware:

const byte addr[] = "EMBA1";

radio.openWritingPipe(addr);

// Wearable B firmware:

const byte addr[] = "EMBB2";

radio.openWritingPipe(addr);

// Base Station firmware:

const byte addrA[] = "EMBA1";

const byte addrB[] = "EMBB2";

radio.openReadingPipe(1, addrA);

radio.openReadingPipe(2, addrB);

radio.startListening();

## 8.4 Collision Avoidance

With two wearables transmitting to one receiver, add a 250ms startup delay in Node B’s setup() to stagger transmissions. Each node transmits every 500ms, so they interleave naturally. The RF24 library’s auto-retry handles any remaining collisions.

## 8.5 Gate 1 — Radio Test (MUST PASS BY HOUR 3)

This is the single most critical milestone. If radios don’t communicate, nothing else matters.

- Wire NRF24L01 to Nano A (wearable) with capacitor. Upload transmit-test: sends a counter every second.

- Wire NRF24L01 to Nano B (base) with capacitor. Upload receive-test: prints received values.

- Open Serial Monitor on Nano B at 115200. Verify counter values appear.

- Walk Nano A away. Note max reliable distance. Must reach 15m minimum.

- If NO data: check MOSI/MISO not swapped, capacitor present, same channel + address, matching CE/CSN pins in code (9, 10).

Do not wire any sensors until this gate passes.

# 9. 3D Printed Enclosures

Print enclosures for the two wearable nodes only. The base station sits on a table and doesn’t need a case.

## 9.1 Design

- Internal dimensions: Measure your mini breadboard + components. Typical: ~52mm x 40mm x 25mm (with components). Add 5mm clearance each side.

- USB slot: Opening on one end for Mini-B USB cable to power bank.

- GPS antenna window: Thin wall or opening on top so the ceramic antenna can see satellites.

- Vent holes: 4–6 small holes on sides.

- Strap slots: Two horizontal slots on the back for a velcro strap.

- Snap-fit lid: Removable top for accessing the breadboard.

## 9.2 Print Settings

- PLA, 0.2mm layer height, 15–20% infill.

- Estimated 1.5–2.5 hours per case. Start the first print during Hour 1 while firmware is being written.

- Use TinkerCAD (free, browser-based) for quick design.

# 10. 24-Hour Schedule & Task Assignments

## 10.1 Team Roles by Discipline

EE (Electrical Engineering): All physical hardware: soldering GPS headers, breadboard wiring, capacitor placement, 3D enclosure design/printing, battery management, hardware debugging with multimeter.

CE (Computer Engineering): All embedded firmware: Arduino IDE setup, RF24/TinyGPSPlus/IMU libraries, wearable firmware, base station firmware, radio configuration, serial protocol definition. Bridges EE and CS — writes test sketches as each sensor is wired, defines the JSON format the dashboard consumes.

CS (Computer Science): Entire laptop browser dashboard: HTML/CSS/JS, Web Serial API, floor plan rendering, GPS-to-pixel calibration, marker animation, status panels, activity log, UI polish. Works independently using mock data for the first 9 hours.

## 10.2 Detailed Schedule

| Time | EE (Electrical Eng.) | CE (Computer Eng.) | CS (Computer Science) |
| --- | --- | --- | --- |
| Hr 0–1 | Solder header pins onto GPS #2. Full inventory check. Test all 3 Nanos power on via USB. Begin 3D enclosure design in TinkerCAD. | Set up Arduino IDE: install Nano board support, RF24 (TMRh20), TinyGPSPlus, MPU6050/Wire libraries. Create folder structure. Write I2C scanner and SPI test sketches. | Create index.html with layout. Implement Web Serial API connection: Connect button, port selection, byte reader, line buffer, JSON parser. Test with mock data generator. |
| Hr 1–3 | Wire NRF24L01 + capacitor to Nano #1 (wearable A). Wire NRF24L01 + capacitor to Nano #3 (base station). Hand both to CE. Start first 3D print. | Flash transmit test to wearable Nano, receive test to base Nano. Configure: channel 108, 250Kbps, PA_MAX, retries(15,15), CE=D9, CSN=D10. GATE 1: Radio works at 15m+. | Build floor plan upload. Draw image on canvas. Build mock data generator (2 fake firefighters). Build calibration mode (click 2 points, enter GPS coords, compute transform). |
| Hr 3–5 | Wire IMU to wearable Nano #1 (A4/A5). Wire GPS to wearable Nano #1 (D3/D4). Hand to CE for testing. Take board near window for GPS fix. | Write IMU heading code (Wire library + MPU6050 registers or Adafruit library). Write GPS reader (SoftwareSerial on D3, TinyGPSPlus). GATE 2: Heading + lat/lng on Serial Monitor. | Build firefighter markers: coloured circles with heading arrows, smooth lerp animation, stale-GPS pulsing. Test with mock data — markers should move smoothly on the canvas. |
| Hr 5–7 | Wire buzzer to wearable #1 (D6). Begin wearable #2: Nano #2 on mini breadboard, wire NRF24L01+cap, IMU, GPS #2, buzzer. Check 3D print; start second if ready. | Write combined wearable firmware: all sensors + radio transmit every 500ms. Flash to wearable #1. GATE 3: Base Nano Serial Monitor shows full JSON from wearable #1. | Build sidebar status cards: node ID, GPS coords, heading with compass direction, signal quality, battery bar, alert status. Add “Send Alert” button writing BUZZ:N to serial. |
| Hr 7–9 | Complete wearable #2 assembly. Wire base station buzzer (D6). All hardware complete at this point. | Write base station firmware: open both reading pipes (EMBA1, EMBB2), receive SensorPacket, construct JSON, Serial.println() at 115200. Add buzzer on incoming BUZZ command from laptop serial. Add bidirectional relay (BUZZ cmd from laptop → radio TX to wearable). | Build activity log panel. Add connection status indicator. Add locate-node highlight animation. Test all UI interactions with mock data. |
| Hr 9–11 | INTEGRATION: Connect base Nano to laptop USB. Debug hardware issues. Mount wearable #1 in 3D case. | Flash final firmware to all 3 Nanos. Coordinate with CS on JSON format. GATE 4: Dashboard shows live positions from both wearables. | Switch from mock to real serial data. Debug coordinate mapping with CE. Fix any parsing or display issues. Add stale GPS indicator. |
| Hr 11–13 | Mount wearable #2 in case. Prepare battery packs. Attach velcro straps. Tidy base station wiring. | Add stationary detection (IMU no-movement > 60s → alertFlag=1). Fine-tune radio timing. Begin ESP-01 WiFi bonus (Section 12) if time allows. | Polish UI: EMBER branding, dark fire-themed styling, responsive layout. Add last-update timestamps. Add compass direction labels. |
| Hr 13–15 | FULL WALK TEST. Wear both nodes, walk demo area. Range test at 20m. Report issues. | Fix firmware bugs from walk test. Tune stationary threshold. Test full alert round-trip. Continue ESP-01 bonus if started. | Fix UI issues from walk test. Ensure markers track correctly. Final polish. |
| Hr 15–18 | Prepare demo area. Print replacements if needed. Charge batteries. Clean up. | CODE FREEZE hour 17. Write cheat sheet: channel 108, addresses EMBA1/EMBB2, baud 115200, pins CE=D9 CSN=D10. | CODE FREEZE hour 17. Final UI check. Pre-load floor plan. Clear test data. |
| Hr 18–21 | Set up demo station. Final power-on test. Prepare backup wiring. | Lead demo rehearsal. Practice transitions. Prepare Q&A talking points. | Prepare dashboard for demo. Pre-calibrate. Practice dashboard walkthrough. |
| Hr 21–24 | Support demo. Fix wires, swap batteries. | Co-present: embedded architecture, radio comms, firmware features. | Co-present: live dashboard demo, GPS calibration, alert system, future expansion. |

## 10.3 Gate Milestones

- Gate 1 (Hour 3): Two NRF24L01 modules communicate at 15m+. Non-negotiable.

- Gate 2 (Hour 5): IMU heading + GPS lat/lng on Serial Monitor.

- Gate 3 (Hour 7): Base Nano receives and prints full JSON from wearable #1.

- Gate 4 (Hour 11): Dashboard shows live positions from serial data. Full integration.

# 11. Testing & Demo Checklist

## 11.1 Pre-Demo Checklist

- Power on Wearable A. Buzzer beeps once (firmware confirmation).

- Power on Wearable B. Buzzer beeps once.

- Power on base station via USB to laptop.

- Open dashboard in Chrome. Click Connect. Select Nano’s COM port.

- Verify connection indicator turns green.

- Verify FF1 and FF2 status cards populate.

- Upload floor plan. Calibrate with 2 points.

- Verify markers appear at correct positions.

- Walk Wearable A around. Verify marker moves.

- Walk Wearable B around. Verify independent movement.

- Stop Wearable A for 60s. Verify stationary alert (buzzer + dashboard).

- Click Send Alert for FF2 on dashboard. Verify Wearable B buzzer activates.

- Disconnect Wearable A power. Verify dashboard shows signal loss.

## 11.2 Demo Script (5 Minutes)

- Intro (30s): “This is EMBER — real-time firefighter tracking for incident commanders.”

- Hardware (60s): Hold up wearable node. Point out each component. Explain NRF24L01 radio link.

- Live tracking (90s): Two team members walk with wearables. Dashboard updates in real time.

- Alerts (60s): One person stops. Stationary alert triggers. Click Send Alert from dashboard.

- Architecture (30s): Wearable → NRF24L01 → base station → USB → browser. No internet needed.

- Future (30s): BMP280 for multi-floor, LoRa for 2km range, WiFi for phone access (show ESP-01 bonus if completed).

# 12. Bonus Feature: ESP-01 WiFi Dashboard

Attempt this ONLY after the core system (Sections 1–11) is fully working and tested. This is an add-on for extra innovation points. If it doesn’t work, it does not affect the core system.

## 12.1 Concept

One ESP-01 at the base station creates a WiFi access point (hotspot) and runs a tiny web server. Anyone in range can connect their phone to the hotspot, open a browser, and see a live firefighter status page — no USB cable or laptop required. This is impressive for judges because it demonstrates wireless accessibility: an incident commander could view the dashboard on a phone or tablet.

## 12.2 How It Works

- Physical connection: The base station Nano sends the same JSON data it sends to the laptop ALSO on a SoftwareSerial pin (D3) to the ESP-01’s RX pin. The ESP-01 reads these JSON lines and stores the latest data for each firefighter.

- WiFi AP mode: The ESP-01 creates a WiFi network called “EMBER-DASH” (no password, or a simple password for judges to type). It runs in AP mode — no router or internet needed.

- Web server: The ESP-01 runs a simple HTTP server on port 80. When a phone connects to the WiFi and opens a browser to 192.168.4.1, the ESP-01 serves a minimal HTML page showing: firefighter names, last known GPS coords, heading, battery %, and alert status. The page auto-refreshes every 2 seconds.

- No floor plan: The ESP-01 has only ~80 KB of usable RAM and cannot serve large floor plan images or run the full canvas-based dashboard. The phone page shows a simple text-based status table. The full visual dashboard remains on the laptop.

## 12.3 ESP-01 Wiring

| ESP-01 Pin | Connect To | Notes |
| --- | --- | --- |
| VCC | 3.3V (from Nano’s 3.3V pin) | ESP-01 is 3.3V ONLY. Shares the 3.3V rail with the NRF24L01. The Nano’s 3.3V regulator can supply ~150mA total. NRF24L01 uses ~15mA, ESP-01 uses ~70mA in AP mode. Total ~85mA — within limits. |
| GND | GND | Common ground. |
| RX | Nano D3 (via voltage divider) | Receives JSON data from Nano. CRITICAL: Nano D3 outputs 5V but ESP-01 RX is 3.3V. Use a voltage divider: 1K resistor from D3 to ESP RX, 2K resistor from ESP RX to GND. This drops 5V to ~3.3V. |
| TX | Not connected (or Nano D2 for debugging) | ESP-01 TX output. Not needed for this application. |
| CH_PD (EN) | 3.3V (via 10K pullup) | Must be HIGH for the ESP-01 to run. Connect to 3.3V through a 10K resistor. |
| GPIO0 | 3.3V (via 10K pullup) | Must be HIGH for normal run mode. Pull LOW only when flashing firmware. |
| GPIO2 | Not connected | Leave floating or pull HIGH. |
| RST | 3.3V (via 10K pullup) | Prevent accidental resets. Connect to 3.3V through a 10K resistor. |

## 12.4 ESP-01 Firmware Overview

The ESP-01 firmware is a separate Arduino sketch using the ESP8266 board package. It does three things:

- Serial.begin(115200): Reads JSON lines from its RX pin (receiving from Nano D3 via SoftwareSerial).

- WiFi.softAP("EMBER-DASH"): Creates a WiFi access point.

- ESP8266WebServer: Serves a simple HTML page at 192.168.4.1 showing the latest data for FF1 and FF2. The page includes a <meta http-equiv="refresh" content="2"> tag for auto-refresh.

## 12.5 Flashing the ESP-01

The ESP-01 requires a USB-to-serial adapter (FTDI or CH340) with 3.3V logic for flashing. During flashing, GPIO0 must be pulled LOW. Steps:

- Install ESP8266 board package in Arduino IDE (Preferences → Boards Manager URLs → add http://arduino.esp8266.com/stable/package_esp8266com_index.json).

- Select board: “Generic ESP8266 Module”, flash size: 1M (no SPIFFS), upload speed: 115200.

- Connect ESP-01 to USB adapter: VCC→3.3V, GND→GND, TX→RX, RX→TX, GPIO0→GND (for flash mode).

- Upload the firmware sketch.

- Disconnect GPIO0 from GND (return to run mode).

- Wire into the base station per Section 12.3.

## 12.6 Time Estimate

If the CE person has experience with ESP8266: 2–3 hours including testing. If no ESP8266 experience: 4–5 hours, with significant risk of debugging WiFi/serial issues. Only attempt this after Gate 4 is passed and the core system is stable (typically after Hour 13).

# 13. Future Expansions

## 13.1 Multi-Floor with BMP280

A BMP280 barometric pressure sensor (~$2) detects floor changes with ~0.3m accuracy. Wires to I2C (same bus as IMU, address 0x76 — no conflict). The base station BMP280 records ground-floor reference pressure, and each wearable’s BMP280 reading is compared to estimate floor number.

## 13.2 Extended Range

- NRF24L01+PA+LNA: Drop-in replacement with external antenna. 100–200m indoor range. Same pins, same library.

- LoRa (SX1276): 2–10 km range. Uses SPI. Ideal for real deployment.

## 13.3 Upgrade to ESP32 or Arduino Nano ESP32

For future versions, upgrading to an ESP32-based board (like the Arduino Nano ESP32) would provide: dual-core processing (eliminating SoftwareSerial timing issues), built-in WiFi and Bluetooth (eliminating the need for separate ESP-01 modules), more RAM (520 KB vs 2 KB), and hardware serial ports for GPS without SoftwareSerial limitations.

## 13.4 Heart Rate Monitoring

A MAX30102 pulse oximeter (I2C, address 0x57) could be added to each wearable for real-time heart rate data. No pin conflict on the Nano.

# 14. Troubleshooting Guide

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| NRF24L01 prints “HARDWARE FAIL” | Wiring error: MOSI/MISO swapped, or missing connection | Double-check: MOSI→D11, MISO→D12, SCK→D13, CE→D9, CSN→D10. Verify VCC is on 3.3V not 5V. |
| Radio init OK but no data received | Mismatched channel, address, or data rate | Both Nanos must use: same channel (108), same addresses (EMBA1/EMBB2), same data rate (250KBPS). Print radio.printDetails() on both. |
| Radio works at 2m but fails at 10m | Missing capacitor on NRF24L01 | Add 10µF capacitor directly across VCC and GND of the NRF24L01. This is required. |
| GPS shows 0.000000 | No satellite fix (indoor, cold start, antenna down) | Take node outside or to a window. Wait 60s. Ensure antenna faces upward. Check SoftwareSerial is on correct pins (D3/D4). |
| GPS data corrupts NRF24L01 packets | SoftwareSerial disables interrupts during GPS reads | Increase radio retries: setRetries(15,15). Or read GPS only every other loop iteration to reduce interrupt conflicts. |
| IMU returns all zeros | I2C address mismatch or wiring error | Run I2C scanner. Check A4=SDA, A5=SCL. Try both 0x68 and 0x69. |
| Nano crashes/resets | 3.3V rail overloaded by NRF24L01 + ESP-01 | Use capacitor. If also running ESP-01, consider external 3.3V regulator (AMS1117-3.3). |
| Serial Monitor shows garbled text | Baud rate mismatch | Serial Monitor baud must match Serial.begin() in firmware. Use 115200 for both. |
| Web Serial shows no ports | Wrong browser or missing USB driver | Use Chrome (not Firefox). Install CH340 driver for your Nano’s USB chip. |
| Dashboard receives garbled JSON | Baud rate mismatch between Nano and Web Serial | Ensure Nano firmware: Serial.begin(115200). Dashboard: port.open({baudRate:115200}). |
| Markers in wrong position | Calibration error | Re-calibrate with points further apart (20m+ ideal). Ensure lat/lng entered with 4+ decimal places. |
| Buzzer silent | Active vs passive confusion, or wrong pin | Test: connect buzzer directly to 3.3V. If it sounds, it’s active. Check D6 in firmware. |
| Second wearable not showing | Pipe address mismatch | Verify Node B uses EMBB2 (not EMBA1). Verify base opens both reading pipes 1 and 2. |
| ESP-01 won’t connect to WiFi | GPIO0 or CH_PD not pulled HIGH | Ensure CH_PD and GPIO0 are pulled to 3.3V via 10K resistors in run mode. |
| ESP-01 receives garbled data from Nano | 5V serial from Nano to 3.3V ESP-01 RX | Must use voltage divider: 1K from Nano D3 to ESP RX, 2K from ESP RX to GND. |